CIST 0165 Networking I

Quiz 1 and Mid-Term Study Guide

Please familiarize yourself with the below guidelines and topics for the upcoming quiz and mid-term exams.

- Guidelines

- Lecture slides, notes, homework and reading assignments will be good reference material.

- Exams are open book, open notes but searching on web for answers and working with others on the exam questions are not allowed.

- Any act of academic dishonesty would directly lead to an F grade in the course.

- Exam questions would be a mix of multiple choice, matching, short answer/definition, and command execution (terminal, cisco packet tracer etc).

Types of Computer Networks

- Local Area Network (LAN)

- Office, School, Colleges

- Wired or Wireless

- Wide Area Network (WAN)

- Connect the computers placed in one location to another location

- Metropolitan Area Network (MAN)

- Designed to be used in a city or a town through the wireless medium or using optical fiber cables

- Campus Area Network (CAN)

- Designed to be used by a particular campus or a military base with computers placed within a limited area

- Home Area Network (HAN)

- Connected within user home that connects various digital devices over a network for communicating with other devices

- Key Concepts

- What is a network

- Infrastructure for computer devices to communicate with each other

- Refers to the connection of two or more computers by some type of medium wired or wireless

- Differences between internet, intranet, and extranet

- Intranet

- Private network contained within an enterprise

- Internet

- Worldwide public internetwork

- Uses protocols such as TCP/IP and HTTP to transfer and view information

- Extranet

- Allows limited and controlled access to internal resources by outside users

- Sources of packet delay

- Malware, Denial of Service, Packet Sniffing, IP Spoofing

- Malware

- Malware can get in host from:

- Virus: self-replicating infection by receiving/executing object (e.g. e-mail attachment)

- Worm: self-replicating infection by passively receiving object that gets itself executed

- Spyware malware can record keystrokes, web sites visited, upload info to collection site

- Infected host can be enrolled in botnet, used for spam or distributed denial of service (DDos) attacks

- Classification of network media

- Common IEEE Ethernet Standards (lecture 3, slide 6)

- Straight vs Crossover Cables

- Copper cables vs fiber optic cables

- Copper cables can only carry signals about 100 meters, while some single-mode fiber optic cables can carry more data up to 25 miles. Fiber optic cable also carries data with less attenuation or signal loss—only about three percent every 100 meters—than copper, which loses over 90 percent over the same distance.

- Usages of NIC, Repeater, Hub, Switch, Router, Modem, Firewall

- Network Interface card (NIC)

- To create and mediated the connection between a computer and the networking medium

- Date is represented as it signals that NIC transmits or receives

- Contains electronic components needed to send and receive signals

- Allows a PC to communicate with computer hardware

- Uniquely identifies a computer

- Networks won’t function correctly if duplicate MAC addresses exist

- Most NICs are built into a computer’s motherboard

- MAC address is stored in nonvolatile memory on the NIC

- Repeater

- The number of nodes on a network and the length of cable used to influence the quality of communication on the network

- Attenuation

- The degradation of signal clarity

- Repeaters

- Work against attenuation by repeating signals that they receive on a network

- Typically cleaning and regenerating the digital transmission in the process

- Amplifies or reshapes a weak signal into its original strength and form

- Hub (Multiport Repeaters)

- A multiport repeater is just a repeater with several ports to which you can connect cabling

- Also referred to as a HUB

- Receives bit signals generated from a connected computer on one of its ports

- Cleans the signal by filtering out electrical noise

- Regenerates the signal to full strength

- Generic connection device used to tie several networking cables together to create a link between different stations on a network

- Active hubs

- Amplify or repeat signals that pass through them

- Passive hub

- Merely connects cables on a network and provides no signal regeneration

- Network Bandwidth

- Amount of data that can be transferred during a specific interval

- Usually measured in bits per second (bps)

- Networks operate from 10 million bps (10 Mbps) up to 10 gigabits per second (Gbps)

- Hub share bandwidth with all other connected computers

- Only one computer can successfully transmit data at a time

- Switches

- Filters and directs network traffic

- Creates subnetworks from a larger network

- A network switch looks just like a hub

- However, a switch reads data in the message, determines which port the destination device is connected to, and forwards the message to only that port

- Data is sent onto the medium one frame at a time

- Each frame has the destination MAC address and the source computer’s MAC address

- When the frame reaches a switch, the switch reads both addresses:

- Keeps a record of which computer is on which port

- Forwards the frame to the port where the destination MAC can be found

- A switch maintains a switching table of MAC addresses that have been learned and their associated port numbers

- Basic Switch Operation

- The switch receives a frame.

- The switch reads the source and destination MAC addresses.

- The switch looks up the destination MAC address in its switching table.

- The switch forwards the frame to the port where the computer owning the MAC address is found.

- The switching table is updated with the source MAC address and port information.

- Advantages

- Switches increase available network bandwidth

- Switches reduce the workload on individual computers

- Switches increase network performance

- Networks that include switches experience fewer frame collisions because switches create collision domains for each connection) a process called micro segmentations)

- Switches connect directly to workstations

- Disadvantages

- Switches are significantly more expensive than bridges

- Network connectivity problems can be difficult to trace through a switch

- Broadcast traffic may be troublesome.

- Switch remembers MAC addresses of recent senders on each port

- If packet is addressed to an unknown address, broadcast to all ports

- Routers

- Routers connect LANs together to create an internetwork

- Typically have two or more network ports to which switches or hubs are connected

- Routers are devices that enable multiple LANs to communicate with one another by forwarding packets from one LAN to another

- Default Routes

- Default Routes tell the router where to send a packet when the router doesn’t have an entry in its routing table

- Network Unreachable

- Network unreachable is a message a router might send to the sending station when the network can’t be found

- Navigates packets across large networks

- Uses the most efficient route

- Similar to switches in that they segment a network and filter traffic

- Routers use the logical address

- Routers use the IP address to route packets to the correct network segment

- Advantages

- Can connect different network architectures, such as Ethernet and Token Ring

- Can choose the best path across the internetwork using dynamic routing techniques

- Reduce network traffic by creating collision domains

- Disadvantages

- Routers work only with routable network protocols; most but not all protocols are routable

- Routers are more expensive than other devices

- Dynamic router communications (inter-router communication) cause additional network overhead, which results in less bandwidth for user data

- Routers are slower than other devices because they must analyze a date transmission from the Physical through the Network layer

- Modems

- Digital to analog

- Analog to digital

- Internet reads analog

- Computers use digital encoding

- Router comes in after the modem

- In a SOHO, a modem embedded in a router

- Types of Modems

- Cable

- Connected using co-axial cable

- Cable is typically provided by the same company that provides cable connection

- Comcast

- DSL

- Connected using typical phone line

- ATT

- Lot of times when the service provider sends a modem – a combination of modem and router – wireless router

- Firewall

- Filters info that comes in from network/internet

- Creates a safety barrier between a private network and the wider external internet

- To avoid disruptions from unwanted elements

- Especially imp for a large org that has lot of computer and servers

- Types of Firewalls

- Host-based firewall

- Software firewall that is installed on a computer

- Protects only that computer

- Ex: Microsoft windows firewall, anti-virus programs

- Network-based firewall

- Combination of hardware and software

- Placed between a private network and the internet

- Protects the entire network unlike the host based by following the management rules that ate applied to entire networks

- Can be a standalone product – used by large organizations

- Can also be built into routers – used by small organizations

- Or deployed in the service provider’s cloud infrastructure – cloud firewall

- Lot of organizations will use both network-based and host-based firewalls

- Ensure maximum protection – 2-layer security

- Source Address Table

- Ethernet switches give every conversation the full 
bandwidth of the network

- Maintains Source Address Table (SAT)

- A switch copies the source MAC addresses and builds a table of MAC addresses of each connected computer

- Wireless Ethernet Standards (lecture 3, slide 41)

- Client

- A client can be a workstation running a client OS or it can refer to the network software on a computer that requests network resources from a server

- Server

- A computer becomes a server when software is installed on it that provides a network service to client computers

- What is a peer-to-peer network

- Computers on a peer-to-peer network can take both a client and a server role

- Any user can share resources on his/her computer with any other user’s computer

- Every user must act as the administrator of his/her computer

- Can give everyone else unlimited access to their resources or grant restricted access to other users

- Usernames and passwords (credentials) are used to control that access

- In a P2P network, no computer has more authority than another; each computer controls its own resources and communicates directly with other computers

- Advantages

- Simple configuration

- Less expensive compared to other network models

- Disadvantages

- Must remember multiple sets of credentials to access resources on several computers

- Desktop PCs and the OS installed on them aren’t made to provide network services as efficiently as dedicated network servers

- Data organization

- If every machine can be a server, how can users keep track of what information is stored on which machine?

- Not scalable

- Not necessarily secure

- Not practical for large installations

- What is the client/server network

- Severs hosts the resources for the clients to use and provide security

- A Client is the computer that requests resources from the server

- Client-Server applications

- Network services are the resources a network makes available to its users

- It includes applications and the data provided by these applications

- In client-server applications

- A client computer requests data or service from a second computer, called the server

- Server/Domain-Based Model

- Server-based networks allow centralized control over network resources

- Users log on to the network with a single set of credentials maintained by one or more server OS

- In most cases, servers are dedicated to running network services and should not be used to run user applications

- Network Topologies – Bus, ring, Star, Hybrid, Star

- Every network provides a method of getting data from one system to another

- Cabling

- Wireless methods

- Standards ensure networking equipment works well together

- Topology describes how parts of a whole work together

- Physical topology

- Network’s hardware and how computers, other devices, and cables work together to form the physical network

- Logical topology

- Path data travels between computers on a network

- Physical Bus Topology

- A continuous length of cable connecting one computer to another in daisy chain fashion

- The simplest and at one time the most common method for connecting computers

- Bus Topology

- Single bus cable

- Connects all computers in a line

- Data flows form each computer onto the bus

- Termination required at end prevent signal from reflecting at the ends of the cable

- Uses a single cable or conductor to connect all nodes

- Used as a backbone to link other topologies

- Ring Topology

- Connects all computers on the network in ring

- Data flows form one computer to next one in circle

- No end of cable and no need for termination

- Single cable

- Runs continuously from node to node

- One problem: Entire network stops working if the cable is broken at any point

- Star Topology

- One central connection for all computers

- Uses a central device (hub or switch) to connect computers

- Each computer has a single length of cable going from its NIC to the central device

- Has fault tolerance

- If one cable breaks, other computers can still communicate

- Not successful early on

- More expensive than bus and ring

- Difficult to redesign early bus and ring hardware

- Advantages

- Much faster technologies than bus

- Using a central device allows for monitoring and managing a network Hubs and switches can include software that collects statistics about network traffic patterns and detect errors

- If cabling and NICs support it, a star network can be easily updated by replacing the central device

- When the number of workstations you need exceed the number of ports on a central device you simply add another central device

- The main disadvantage is that the central device represents a single point of failure

- If the hub or switch fails or someone kick the power cord out of the outlet, the entire network goes down

- Mesh Topology

- Hybird Topology

- A hybrid topology is mixture of topologies

- Tree Topology

- Two or more-star topologies connected by a common backbone

- LAN vs WAN

- LAN

- A LAN (local area network) is usually contained in a small space

- This LAN has five computers, a network printer, a local printer, a scanner, and a switch, and uses a star topology

- WAN

- A WAN (wide area network) is a group of LANs that spread over a wide geographical area

- Spans multiple geographic areas and is usually connected by common telecommunication carriers

- LAN vs WANS

- LANS

- Your Control

- Your Equipment

- Your premises

- WANS

- Outside Control

- Outside Entity’s Equipment

- Outside your physical property

- Root, top-level, authoritative, and subdomain name servers

- International Corporation for assigned names and number

- These store IP addresses of the nameservers for all the top-level domains (TLDs): com, net, uk, edu, cn, etc

- Querying the DNS Hierarchy

- Client wants IP address of cs.pitt.edu

- Query iteratively, starting with the root server

- Each server replies with an answer or the name of server to contact:

- “I don’t know this name but ask this server.”

- Performance problems?

- Every request hits the root server

- Common domains, like google.com are queried billions of tines per day!

- Elements of network protocol

- Encoding – based on medium (wired or wireless)

- Formatting and encapsulation – common format, identifying information

- Timing – flow control, acknowledgement, and re-transmission

- Size – fragmentation, numbering

- Delivery options – unicast, multicast, broadcast

- Functionalities of OSI Model Layers

- OSI Model

- Used as a reference model by vendors

- Introduced to enable communication between different computers

- Implements all the elements of a network protocol

- Modular design

- Easy to troubleshoot

- OSI layer 1, layer 2, layer 3 devices

- TCP/IP Model Layers

- Understand protocol data units, segments, packets, frames

- Protocol data units is a single unit of information transmitted among peer entities of a computer network.

- Packet is a small segment of a larger message

- Frame is data that is transmitted between network points as a unit complete with addressing and necessary protocol control information.

- MAC address vs IP Address

- MAC address

- Found at the Data Link Layer of the OSI model

- Used by bridges and switches to make forwarding decisions within a network or subnetwork

- IP address

- Logical address when TCP/IP is used on an internetwork

- TCP Features

- Multiplexing

- Feature of both TCP and UDP

- Solution for handling traffic for multiple applications simultaneously

- Utilizes ports for identifying traffic

- Creates sockets to track traffic

- Source Socket

- Destination Socket

- End-to-end packet delivery

- Connections terminated on both sides to finish conversation

- Introduces new Flag Bit of “FIN”

- TCP conducts Four Way handshake

- Reliability

- TCP gives reliable data transfer

- Utilizes Sequence and Acknowledge fields of TCP header

- What happens on Error

- Flow control

- Protocol to ensure that the sender does not send data faster than the receiver can receive it

- Receiver sends window size in ACK

- If window is small, sender waits

- Control how much data can be sent before ACK

- Allows Receiving Host to adjust window size up or down

- Congestion control

- IP Addressing and Routing

- Binary to decimal and vice versa

- Different classes of IP Addresses

- Class A

- Reserved for governments and large corporations throughout the world

- Each Class Address supports 16,777,214 hosts

- Class B

- Assigned to large and medium sized companies

- Each Class B address supports 65,534

- Class C

- Addresses are assigned to groups that do not meet the qualifications to obtain Class A or B addresses

- Each Class C address supports 254 hosts

- Class D

- Addresses (also known as multicast addresses) are reserved for multicasting

- Multicasting is sending of steam of data (usually audio and video) to multiple computers simultaneously

- Class E

- Addresses are reserved for research, testing, and experimentation

- The Class E range starts where Class D leaves off

- Private IP ranges

- Many companies use private IP addresses for their internal networks

- Will not be routable on the internet

- Gateway devices have network interface connections to the internal network and the internet

- Route packets between them

- IP Classes

- Subnet Masks

- Indicates how much of the IP address represents the network or subnet

- Standard (default) subnet masks:

- Class A subnet mask is 255.0.0.0

- Class B subnet mask is 255.255.0.0

- Class C subnet mask is 255.255.255.0

- Understand the usage of the below commands and be able to explain the result:

- traceroute or tracert

- Command-line utility that you can use to trace the path that an Internet Protocol (IP) packet takes to its destination.

- ipconfig /all

- Output of the default command contains the IP address, network mask, and gateway for all physical and virtual network adapters.

- Ping

- Used to test the reachability of a host on an Internet Protocol network

- Nslookup

- Cross-platform command for obtaining DNS records

- Whois

- A query and response protocol that is widely used for querying databases that store the registered users or assignees of an Internet resource, such as a domain name, an IP address block or an autonomous system, but is also used for a wider range of other information.

- netstat

- Cisco Packet Tracer

- Basic commands and functionalities (to be explained in class)

**	Sections 2,3,4 would be covered in Quiz 1

***	Sections 2,3,4,5,6 would be covered in the mid-term